Care of MRA BP-111-RS Microwave

Do not use this equipment if you have not been trained.

Always wear gloves.

Put gray duct into hood.

Check power setting: for most tissue it should be at 55, for especially delicate tissue at 45

Set TEMP with bottom timer- ONLY use the arrows to adjust temp, do not touch any of the other keys.

Set TIME with top timer- use arrows to adjust, press circle arrow button to fix time setting.

Start by pressing   F1

RST

Always clean temperature probe. Put probe in vial of citrisolve, 67C, for 5 min, wipe off dissolved paraplast with towel.

Clean up any spills or spilled wax with pan scrapper.