Care of TBS embedding center

Do not use this equipment if you have not been trained.

Always wear gloves.

ONLY use Paraplast Xtra.

Turn on at least 30min prior to use so that it can come to temperature.

Do NOT change temperature settings.

Do NOT turn off cold plate or heating elements.

Turn off the embedding center when finished and clean up molton wax with paper towels.

Clean up solidified wax with pan scrapper.

Clean up water from cold plate and tray under cold plate with paper towels.

Clean up wax from floor with floor-specific pan scrapper to prevent the floor from becoming VERY slippery.

Refill pitchers in 62C oven with Paraplast Xtra chips (fill with chips to top so that when melted, the pitchers are about ½ or ¾ full).

Rotate fresh Xtra pitchers to the back of oven.