Embedding of Tissue

- Turn on embedding center at least 30 minutes before you plan to embed

- Label molds and preheat inside chamber

- Place 4-6 pre-heated molds on hot part of embedding center and pour molten Paraplast Xtra into the molds

- Place individual samples in to each mold using tools provided

- Over the cold spot orientate your sample as desired and place on cold section of center

- Repeat until all samples are embedded

- Leave samples on the cold section until hardened about 30 minutes after the last mold is poured.

- Remove blocks into plastic bags and into the fridge for long-term storage at 4°.

- Turn off center and clean up wax and water.

Mounting Blocks

- Label mounting cassettes and place in container: plastic tray or pipette box

- Remove molds by cutting down the sides and carefully peeling away

- Trim blocks. Remove as much of the bottom as required to obtain a flat surface.

Working one block at a time pour molten wax into cassettes. Place trimmed block into mount in the orientation you desire but it should be as parallel as possible to the bottom edge as possible.

Mounting cassettes

Wax block

Sample

Bottom edge

First part to be cut

5.  Let cool and firm up at least an hour before you section